🏙 URBAN DISTRIBUTED AI SYSTEM

Project Notes — How It Works

Master's Thesis — Distributed Systems — 9 CFU

# 📊 SECTION 1 — What the Metrics Mean

## 1.1 PM2.5 — Particulate Matter

PM2.5 stands for Particulate Matter 2.5 micrometers. These are tiny particles in the air — smaller than a human hair — that come from vehicle exhaust, factory smoke, and burning fuel. They are dangerous because they are small enough to enter the lungs and bloodstream.

| PM2.5 Level (µg/m³) | Severity | What it means | System Action |
| --- | --- | --- | --- |
| 0 – 35 | LOW | Clean air — safe for everyone | Normal traffic |
| 35 – 75 | MEDIUM | Moderate — sensitive people affected | Normal traffic (monitor) |
| 75 – 150 | HIGH | Unhealthy — everyone affected | Reduce traffic |
| 150+ | CRITICAL | Hazardous — health emergency | Close road |

📌 Source: WHO (World Health Organization) Air Quality Guidelines 2021

## 1.2 Visibility (km)

Visibility is how far you can clearly see in the atmosphere, measured in kilometres. Low visibility is caused by fog, smog, dust, or heavy pollution particles. Low visibility is dangerous for drivers because they cannot see road hazards, traffic lights, or other vehicles in time.

| Visibility | Condition | Risk Level | System Action |
| --- | --- | --- | --- |
| 10+ km | Clear | None | Normal traffic |
| 5–10 km | Hazy | Low | Monitor |
| 2–5 km | Foggy | Medium | Reduce traffic |
| < 2 km | Dense fog | HIGH | Close road immediately |

## 1.3 NOx — Nitrogen Dioxide

NOx (Nitrogen Oxides) are gases produced mainly by car engines, power plants, and industrial processes. NOx causes respiratory problems and contributes to forming ground-level ozone (smog). It is used in this system as a secondary pollution signal alongside PM2.5.

| NOx Level | Meaning |
| --- | --- |
| 0 – 20 | Normal background level — clean area |
| 20 – 60 | Moderate — busy road or light industry |
| 60 – 150 | High — heavy traffic or industrial zone |
| 150+ | Very high — near factory or highway junction |

## 1.4 Traffic Score

Traffic is a computed score from 0 to 100 representing road congestion and driving difficulty. In this system it is calculated from humidity and wind speed: higher humidity + stronger wind = harder driving conditions. It is used as an input feature to the ML model.

| Traffic Score | Meaning |
| --- | --- |
| 0 – 30 | Low — free flowing traffic |
| 30 – 60 | Moderate — normal urban traffic |
| 60 – 80 | Heavy — congested roads |
| 80 – 100 | Critical — gridlock or emergency |

# ⚙ SECTION 2 — How the Project Works

## 2.1 System Overview

This project is a 3-layer distributed AI system that autonomously manages urban traffic based on pollution and visibility data. It runs entirely on one PC using Docker containers. Each container is a separate 'computer' that communicates with others only through MQTT messages — like a real distributed city network.

## 2.2 The Three Layers

EDGE LAYER — edge_agent.py

The Edge layer is closest to the physical world. It simulates IoT sensors and local AI agents at each city zone.

- What it is:  Runs inside one Docker container

- Coverage:  Manages all 3 zones: Industrial Zone, Residential Zone, Green Park

- Input:  Reads PM2.5, visibility, NOx, traffic every 1 second

- ML model:  Runs a Decision Tree ML model trained on 136,438 real rows

- Output:  Publishes decisions to topic: city/decisions

Decision logic:

IF PM2.5 > 150 OR visibility < 2km  →  CRITICAL  →  Close road

IF PM2.5 > 75  OR visibility < 5km  →  HIGH      →  Reduce traffic

IF PM2.5 > 35                        →  MEDIUM    →  Normal traffic (monitor)

ELSE                                 →  LOW       →  Normal traffic

FOG LAYER — fog_coordinator.py

The Fog layer sits between Edge and Cloud. It aggregates data from all zones and detects district-wide emergencies that individual edge agents cannot see alone.

- What it is:  One Docker container — district coordinator

- Input:  Listens to city/decisions from all 3 edge zones

- Logic:  Counts how many zones are HIGH or CRITICAL

- Hotspot:  If 2+ zones in danger → HOTSPOT detected → issues district command

- Output:  Publishes summary to cloud via city/fog/summary

Hotspot logic:

IF 2+ zones CRITICAL  →  EMERGENCY  →  Close all roads in affected zones

IF 2+ zones HIGH      →  RESTRICT   →  Reduce to 30% traffic flow

ELSE                  →  NORMAL     →  No district action needed

CLOUD LAYER — cloud.py

The Cloud layer is the global brain. It receives summaries from the Fog layer, tracks city-wide statistics, and broadcasts a global policy to all layers.

- What it is:  One Docker container — global orchestrator

- Input:  Receives fog summaries and edge decisions

- Logic:  Updates global policy: NORMAL / ALERT / EMERGENCY

- Output:  Broadcasts policy to all layers via city/cloud/policy

Policy logic:

IF avg PM2.5 > 150 OR hotspot CRITICAL  →  EMERGENCY  (PM2.5 limit drops to 50)

IF avg PM2.5 > 75  OR hotspot HIGH      →  ALERT      (PM2.5 limit drops to 65)

ELSE                                    →  NORMAL     (PM2.5 limit stays at 75)

SENSOR LAYER — sensor.py

The Sensor layer simulates real IoT sensors in each zone. It generates pollution data independently for each zone based on realistic profiles.

| Zone | Profile | Base PM2.5 | Spike Chance | Recovery |
| --- | --- | --- | --- | --- |
| Industrial Zone | Factory area — high pollution | 95 µg/m³ | 30% per cycle | Slow (0.15) |
| Residential Zone | Homes — medium pollution | 35 µg/m³ | 15% per cycle | Medium (0.30) |
| Green Park | Nature area — low pollution | 12 µg/m³ | 5% per cycle | Fast (0.60) |

ADMIN DASHBOARD — app.py + index.html

The Dashboard is a Flask web application that subscribes to all MQTT topics and pushes live updates to the browser every second via WebSocket (Socket.IO).

- Access: Open browser at: http://localhost:5000

- Edge data: Shows all 3 zone cards with PM2.5, visibility, NOx, traffic, severity, decision

- Fog data: Shows fog hotspot level, command, affected zones, avg PM2.5

- Cloud data: Shows cloud policy, total decisions, road closures, hotspots caught

- Log: Live event log showing all layer events in real time

- Trends: PM2.5 trend arrows (↑ rising, ↓ falling, → stable)

# 📡 SECTION 3 — MQTT Communication

MQTT (Message Queue Telemetry Transport) is a lightweight messaging protocol designed for IoT devices. Devices publish messages to topics. Other devices subscribe to topics they care about. This is exactly how real smart city sensors communicate.

| Topic | Who Publishes | Who Subscribes | What it carries |
| --- | --- | --- | --- |
| city/all | Sensor | Edge agents | Raw sensor readings (PM2.5, visibility, NOx, traffic) |
| city/decisions | Edge agents | Fog, Cloud, Dashboard | Zone decision + severity + all metrics |
| city/fog/summary | Fog coordinator | Cloud, Dashboard | District hotspot status, avg PM2.5, command |
| city/fog/commands | Fog coordinator | Edge agents | District-wide override commands |
| city/cloud/policy | Cloud | All layers | Global policy: NORMAL / ALERT / EMERGENCY |

# 🤖 SECTION 4 — The Machine Learning Model

The ML model is a Decision Tree Classifier from scikit-learn. It is trained once at startup using the processed_weather.csv dataset derived from the GlobalWeatherRepository (136,438 real rows from cities worldwide).

## 4.1 Training Data

| Feature | Source | Role in model |
| --- | --- | --- |
| pm25 | air_quality_PM2.5 | Main pollution signal |
| visibility | visibility_km | Fog detection |
| traffic | humidity × 0.4 + wind × 0.6 | Driving difficulty |
| nox | air_quality_Nitrogen_dioxide | Secondary pollution |
| action | Computed rule | Target: 1=act, 0=normal |

## 4.2 Action Label Rules

action = 1  (Reduce/Close)  IF:  PM2.5 > 75  OR  visibility < 5  OR  'fog' in condition

action = 0  (Normal)        ELSE: all conditions acceptable

## 4.3 Why Decision Tree?

- Explainable: Simple to explain to thesis examiner — you can draw the tree

- Fast: Fast prediction — runs in microseconds on edge hardware

- Lightweight: No heavy dependencies — no PyTorch, no TensorFlow

- Controlled: max_depth=5 prevents overfitting on 136K rows

# 🐳 SECTION 5 — Docker & Containers

Docker runs each component in an isolated container — like a separate mini-computer. All containers share the same network inside Docker, so they can talk to each other via MQTT using container names as hostnames.

| Container | Image | Port | Role |
| --- | --- | --- | --- |
| mqtt_broker | eclipse-mosquitto | 1883 | MQTT message broker — central hub |
| edge_agent | python:3.11 | — | Edge layer — ML decisions for all 3 zones |
| fog_coordinator | python:3.11 | — | Fog layer — hotspot detection |
| cloud_orchestrator | python:3.11 | — | Cloud layer — global policy |
| sensor_node | python:3.11 | — | Sensor simulator — independent zone data |
| admin_dashboard | python:3.11 | 5000 | Flask dashboard — http://localhost:5000 |

Startup order (important):

mqtt → edge → fog → cloud → sensor → dashboard

Each container waits for the previous ones before starting. This is controlled by depends_on in docker-compose.yml.

# 🚀 SECTION 6 — How to Run the Project

## Step 1 — Process the dataset

python process_dataset.py

Reads GlobalWeatherRepository.csv and creates processed_weather.csv with PM2.5, visibility, NOx, traffic, action columns. Copy the output file into the Edge/ folder.

## Step 2 — Start all containers

docker-compose up --build

Builds all 6 Docker images and starts all containers in the correct order. First run takes 3-5 minutes to download Python image.

## Step 3 — Open the dashboard

http://localhost:5000

Open this URL in any browser. The dashboard updates automatically every 1 second.

## Step 4 — Run evaluation

python evaluate.py

Runs baseline vs agentic comparison on 500 test samples. Prints results table and saves to evaluation_results.csv.

## Step 5 — Stop everything

docker-compose down

# 📁 SECTION 7 — Project File Structure

| File | Location | Purpose |
| --- | --- | --- |
| process_dataset.py | Root | Processes GlobalWeatherRepository.csv → processed_weather.csv |
| docker-compose.yml | Root | Defines and starts all 6 Docker containers |
| mosquitto.conf | Root | MQTT broker config — allows anonymous connections |
| evaluate.py | Root | Thesis evaluation — baseline vs agentic comparison |
| edge_agent.py | Edge/ | Edge layer — ML model + 3 zone decisions |
| processed_weather.csv | Edge/ | Training data for ML model (136K rows) |
| fog_coordinator.py | Fog/ | Fog layer — hotspot detection + district commands |
| cloud.py | cloud/ | Cloud layer — global policy + statistics |
| sensor.py | sensor/ | IoT sensor simulator — independent zone profiles |
| app.py | dashboard/ | Flask + SocketIO server — MQTT → WebSocket bridge |
| index.html | dashboard/templates/ | Admin panel UI — live updates via WebSocket |

Urban Distributed AI System — Master's Thesis Notes